1993 Toyota Land Cruiser, chassis VIN #: HDJ810040324

Mileage: 150,000km

Codename: Sneaky Pete

1HD-T Specs: https://www.engine-specs.net/toyota/1hd-t.html

1HD-T Engine Manual: https://www.manualslib.com/manual/1146213/Toyota-1pz.html
Transmission Code: H151F, has triple ring synchros on 1st and 2nd gear.

| Fluid Type | Brand / Product |
| --- | --- |
| Gear Oil (Diffs) | AMSOIL 75W-140 Gear Lube 2.5L (front), 2.5L (rear) → 5L total |
| Transmission Fluid | ACDelco 10-4014 Friction Modified Synchromesh ~2.5L |
| Transfer Case Fluid | AMSOIL 75W-140 Gear Lube ~4.2L |
| Engine Oil | AMSOIL 10W-30 Diesel Motor Oil ~9L (with filter) |
| Transmission Additive | LUBEGARD Standard Gear & Rear End Oil Additive (3-Pack) |
| Diesel Additive | Stanadyne Diesel Fuel Additive (Lubricity Formula) |

Best FSM Manuals

Liam’s FSM Manuals on Google Doc: https://drive.google.com/drive/folders/1vbfnCAz2kWZ4vJo4dumWCPXyksyl_Cxu

1HD-T FACTORY WORKSHOP MANUAL : https://lc100e.github.io/

TIS ACCESS (North America Only)

username: b1alpha@gmail.com

password: ate-nma4vhy2mtc@AET

Invoices: https://drive.google.com/drive/u/0/folders/1hSkSzHMNy0__jJpomJM_hni7hHG1cJmb

Parts Shopping

- Partsouq Parts List: https://partsouq.com/HDJ810040324

- Megazip Parts: https://www.megazip.net/

- HD Auto: HD Automotive

- Radd Cruisers: Rad Cruisers - Kamloops

Auto Body Shop

- Pro-Tech Collision Centre
1989 1st Ave, Prince George, BC V2L 2Z3
www.protechcollision.ca
(250) 564-9848

## 150,000 km tune-up items (Maintenance Schedule Link)

- assess turbo

- assess existing oil spray

- check gaskets

- assess valve clearance

- air filter

- flush engine coolant

- assess brake lines

- assess brakes

- steering linkage

- steering gear box oils

- front wheel bearing and thrust bush grease

- steering knuckle and chassis grease

- propeller shaft grease

- ball joints

- inspect glow plugs

- inspect injectors

- inspect fuel heater

- new filters (fuel and oil)

- ac pulley

- differential service

- steering pump service

- transfer case service

- timing belt tensioner/pulley & water pump/pulley (done recently)

## Back to factory items (Jan 21st)

- terra-clean

  - fuel system

  - engine oil

  - differentials

  - transfer case

  - clutch

  - trans

  - radiator

  - air cnditioning

- lubegaurd syncro gear oil additive

- ozone interior

- replace fuel heater relay

- upgrade ignition wiring

  - add relay for any non-factory ACC circuit

  - prep head unit for 400w on same relay

  - fix glow plug timer wiring

  - fix turbo timer wiring

- replace injectors seals

- air conditioning refresh

- upgrade engine bay wiring

- upgrade lights to LED

- upgrade intake manifold

- wash valve cover and rusty cover bits

- replace front r/l grab handles

- replace center ash tray

- fix grill plastic

- remove spring spacers

- adjust suspension settings

- fix front differential camber

- install new turbo

- add gauges for temps

- install new front mount intercooler

- big end con-rod bearings

## Body repair items (April)

- new tailgate (scrap yard)

- prepare roof for paint

- choose wrap color (black)

- wrap roof

- reseal wheel flares with RTF

- undercoat with Krown spectrum spray liner

## Builds (examples)

- PDI Build

- All Offroad 4x4

- HD Automotive

- Adventure Corp (AE Co.)

  - 12mm JP tuned Injector Pump

  - 18g Billet wheel turbo

  - DCA front mount intercooler

  - Radius Fab Airbox

  - Radius Fab Snorkel

  - GFB 44mm External wastegate with screamer pipe

## Forums

- IH8Mud: https://forum.ih8mud.com/threads/1-hdt-rebuild-late-2019-start.1123353/

Shifting Issues?

How is your shifter seat and bushing? I thought my synchros were bad on 2 and 3 until I pulled my shifter and found the bushing was broken and the shifter seat brittle, hard and was 70% missing. I just replaced both and shifting problems were all fixed. https://forum.ih8mud.com/threads/h151f-replacing-synchros-on-a-96-j80.1150048/